Project Overview

The goal is to design a compact, high-reliability PCB centered on an STM32 MCU that interfaces with a 6-axis IMU (Inertial Measurement Unit) and a ToF (Time-of-Flight) Range Sensor.

The board serves as the "navigation head" for a rover. It must provide real-time tilt/acceleration data to compensate for uneven terrain and distance data for obstacle avoidance. The system uses a shared I2C bus for simplicity, with dedicated hardware interrupts to ensure the MCU is only alerted when valid data is ready, reducing CPU overhead during high-speed traversal.

Required Components/Interfaces

### 1. Processing & Logic

- MCU: STM32F401RE  (Excellent I2C hardware support and DMA).

- Programming Interface: 4-pin SWD (Serial Wire Debug) header.

### 2. Sensors (I2C Slaves)

- IMU: Bosch BNO055. Chosen for high vibration rejection and integrated step/motion counters.

- Range Sensor: ST VL53L4CD (Long range, up to 4m) with an optical cover glass.

### 3. Connectivity & Power

- Primary Bus: I2C (SDA, SCL) with $2.2k\Omega$ pull-up resistors.

  - Addressing: Need to specify addresses for your devices

- Interrupts: Two GPIO lines (one from IMU, one from ToF) to STM32.

- Connectors: JST-GH (1.25mm pitch) locking connectors to prevent disconnection on rocky terrain.

- Power: 5V Input $\rightarrow$ 3.3V LDO (e.g., AP2112K) to power the logic and sensors.

Electrical Characteristics

| Parameter | Specification | Purpose |
| --- | --- | --- |
| I2C Speed | 400 kHz (Fast Mode) | High-speed polling for tilt compensation. |
| Operating Voltage | 3.3V±5% | Standard logic level for STM32 and sensors. |
| Bus Capacitance | <400pF | Ensures signal rise times meet I2C spec. |
| Decoupling | 0.1μF + 10μF per IC | Filters out low and high-frequency ripples. |
| ESD Protection | USBLC6-2SC6 | Protects I2C lines from static build-up on the rover frame. |

Block Diagrams

The architecture follows a star-topology for power and a bus-topology for data.

- Power Stage: 5V Input -> Filter -> 3.3V Regulator -> Decoupling -> Sensors/MCU.

- Signal Stage: STM32-> I2C Bus (with Pull-ups) ->IMU & Range Sensor.

- Control Stage: Sensors -> Interrupt Lines -> STM32 GPIOs.

References

STM32 I2C Application Note (AN4744): Guidelines for robust I2C communication.

VL53L4CD Datasheet: Distance sensing specs and XSHUT pin timing.

BMI270 Datasheet: Vibration robustness and mechanical mounting requirements.

IPC-2221: Generic standard on printed board design for thermal and vibration management.

Tutorial: https://youtu.be/aVUqaB0IMh4?si=26_mtlfAtesHz-LI